CPU performance:                                                                                                                        CPU performance determined: latency/throughoutput/responding time                                                                 •ThroughPut: the total amount of work done in a given unit of time                                  •Clock Frequency/Rate:  cycles per second, e.g., 3.0GHz = 3.0*10^9Hz, ClockRate = 1/ClockCycle •Clock Cycles per Instruction: CPU time = #Instruction*CPI*CC or /CR Locality:                                                                                                                                                             •Temporal: recently accessed item are likely to be accessed again in the near future(loops, repeated call to same function) •Spatial: Recently accessed items are likely to be in continue ram space(array, sequential operations) •Continually using the same type of instruction is not locality                                                                              Caching:                                                                                                                                                           •Hit: find b in the cache at level k •Miss: not find at level k, so the level k cache must fetch b from lower level k+1 •If level k is full then some block must be replaced            •Mapping: apply modding (i.e., 7mod4=3) to get position. •Replacement: if it is full in mapping step, use either LeastRecentlyUsed or FirstInFirstOut to overwrite contents.  • Cold miss: when the block b does not exist at level k. It occurs while data is caching for the first time. • Capacity miss: when active block is larger than the size of cache     • Conflict miss: when multiple data from level k+1 map to same position in level k due to mapping policy, it may cause trashing                                                                                               Example: 2-way set, 4 bytes cache lines, 4 sets, LRU policy, cache: 4,6,26,12,0,2,21,31. •4=0100 =>index=10, offset=00(cold miss) •6=0110 =>index=10, offset=10(hit),   •26=11010 =>index=10, offset=10(cold miss) •31=11111 =>index=10, offset=11 (hit)                                                                                                                              AMAT Calculation:                                                                                                                                        Example: Main mem access=100ns, 50%instruction require data access, L1miss rate=3.6%, hit time=1.26ns, idealCPI=2.0, L2 miss rate=42%, hit time=21.24ns                                                                                                                                   •AMAT1 =HitTime+(MissRate*Penalty) =1.26+(3.6%*100) =4.86ns •CPIstall1 =CPIideal+AccessRate*MissRate*(MissPenalty/HitTime) =2+3.6%*50%*(100/1.26) =3.43 •CPUTime2 =IC*CPIstall*ClockCycle =IC*3.43*1.26 =IC*4.32 •AMAT2 =L1HitTime+L1MissRate*(L2HitTime+L2MissRate*Penalty) =1.26+3.6%*(21.24+42%*100) =3.54ns •AvgMemStallCycle2 =AccessRate*L1MissRate*(L2HitTime+L2MissRate*L2MissPenalty) =50%*3.6%*(21.24+42%*100/1.26) =0.98 •CPIstall2 =CPIideal+AMSC =2+0.98 =2.98 •CPUTime2 =IC*CPIstall*CC =IC*2.98*1.26=IC*3.75                                                         •CNF: (a+b)*(c+d)*(e+f) •DNF: ab+cd+ef •Functional Completeness: a sets of functions which can describe every operations. {+, *, not} is functional completeness. {+, not} is bothe complete and minimum.

•1 bit half adder: S = A XOR B C = A AND B •1bit full adder: S = (A XOR B) XOR Cin        n-bit full adder: C0in = C1out, connection of 1 bit full adders                                            MUX & DEMUX: n-bit ctrl signal => 2^n options

•D: do nothing, just delay for a cycle •T: if input = 1, toggle current state •SR: S=1 set next state 1, R=1 set next state 0; S=0 and R=0 hold; Cannot have S=1 and R=1 •JK: Same as SR, but toggle if j=k=1. •Registers are just collection of flip-flops, n-bits => n- registers: ParallalInParallalOut, SerialInParallalOut, SISO, PISO                                                                        Min. Clock Cycle = Combinational circuit propagation delay + setup time + clk-to-q •Speedup: Split add and shift into two different tasks / Insert register between to store results temporarily / increase clock frequency                                                                                                                                                                                        MIPS Assembly: RTL                                                                                                                                            • add $8, $9, $10 = R[8] <- R[9] + R [10] • addi $8, $9, 127 = R[8] <- R[9] +127 • lw $13, 32($10) = R[13] <- Mem[R[10] + 32] • sw $13, 8($10) = Mem[R[10] + 8] <- R[13]                                                addui: i stands for immediate values, u stand for unsign(force handling data unsign. addui $t0, $t1, -1 is actually adding the maximum unsigned value due to underflow) *: words and int are 32-bits                                                                                               Instruction Formats:                                                                                                                       •R-type: opcode(6: 000000), rs(5, first param), rt(5, second param), rd(5, destination), shamt(5, shift amount), funct (6, funct code) •I-type: opcode(6), rs(5, source), rd(5, destination), imm(16, offset/value for arithmetic operations)           •J-type: opcode (5, j=000010, jal=000011), address(26, pseudo-direct: [next_PC=(PC&0xF0000000)|(target << 2))], PC-relative jump are bne/beq in I-type)                                                                                                                                              R add $t0, $s1, $s2: 000000|1001|10010|01000|00000|100000                                                                    R sll $s0, $t0, 4: 000000|00000|01000|10000|00100|000000                                                                                                       I addi $t1, $t0, 10: 001000|01000|01001|0000000000001010                                                                  I lw $t1, 12($t0): 100011|01000|01001|0000000000001100                                                                   I bne $t0, $t1, 24: 000101|01000|01001|0000000000011000 <= this is pc-relative

VILW: very long instruction words, encoded multiple instructions into an issue package. E.g., 64bit package=(32bit)+(32bit), one IF stage for one package, multiple ID, EX simultaneous                                                                                     Renaming:  for every instruction which write to a register, create a new value name for the destination.                                                                                   add t6, t0, t2   sub t4, t2, t0 xor t0, t6, t2 and t2, t2, t6          MultiThreading: software, programming model which allows for multiple, concurrent threads execution, each with own context(variables, PC, register value, stack), but share address space heap. Multithread does not require multicore. If no multicore, multithreading could be achieved via time-division multiplexing, requiring context switching. With multicore, threading could run simultaneously.  multicore: hardware, multiprocessor: hardware                                                                                                       Superscalar: multiple instructions within a clock cycle. Multiple ALU in data path running same time.

•Multi cycle data path minimum clock ------------------------------------------------------------ --------------------Single cycle datapath: 800ps clock cycle, pipelined: 200ps clock cycle, uneven time for each stage: ID and WB only 100 ps. Assuming 3 lw instruction above,                Ideal speedup = time between instructions in single cycle/in pipelined = 800/200 = 4 Actual speedup=time complete in single cycle/in pipelined=(3*800)/(7*200)=1.714 Actual speedup is approaching ideal speedup as the number of instructions increase Classic Performance time = IC*CPI*Clock cycle                                                              Time for pipeline = fill time + (IC*clock cycle) => CPI approaching 1                                                            Structural Hazard: two instructions need to use same hardware at same time, Data Hazards: RAW: the only depenciy; RAR: never hazard; WAR: most time not hazard; WAW: out-of-order execution. Forwarding: ALU-ALU, ALU-MEM, MEM-MEM                                   VLIW: •Static Scheduling •In-order execution •single IF but many EX units •Instruction packed together in issue packet by compiler Static SS: •Static scheduling •in-order execution •many IF units(or one IF fetching multiple instr.) and many EX units •Compiler explicitly schedules each “route” through the datapath. Dynamic SS: dynamic scheduling •out-of-order exec. •singe IF unit but many EX units •IF unit might fetch multiple instr. Per cycle